..PROFESSIONAL INSTALL ONLY..

Do NOT give this manual to the end user / home owner

GSM Intercom System

Models 5AB, 5ABK, 5AS, 5ASK, 5HS, 5HSK, 5HB, 5HBK, 5FB, 5FBK, 5IMP, 5IMPK

Manual Version 3

Scan QR code below to download the installer APP

Contents

| Overview of System | …………….Pg 3 |
| --- | --- |
| Site Survey | …………….Pg 3 |
| SIM card | …………….Pg 3 |
| Power | …………….Pg 3 |
| Installation | …………….Pg 4 |
| Architectural panels | …………….Pg 4 |
| Hooded Panels | …………….Pg 4 |
| Flush Panels | …………….Pg 4 |
| Inserting the SIM card | …………….Pg 5 |
| Connections on GSM controller | …………….Pg 5 |
| Output Connections Example | …………….Pg 6 |
| Powering Up | …………….Pg 6 |
| Installing Programmers App for first time | …………….Pg 7 |
| Programming a Brand New Install | …………….Pg 8 |
| Programming an Existing Install | …………….Pg 8 |
| Programming |  |
| Check Reception | …………….Pg 9 |
| Programming Dial Out Numbers | …………….Pg 10 |
| Program Caller ID Numbers | …………….Pg 11 |
| Programming Additional Features | …………….Pg 12 |
| Volumes | …………….Pg 13 |
| Dial Times & Talking Time | …………….Pg 13 |
| Service Calls | …………….Pg 14 |
| Diagnostic Info | .……….Pg 14-15 |
| Pass Codes | …………….Pg 16 |
| Relay Times | …………….Pg 16 |
| Notifications | …………….Pg 17 |
| Keypad Programming | ………….Pg17-19 |
| Auto Relay Trigger Times | …………….Pg 20 |
| Client List on iphone | …………….Pg 20 |
| Client List on Android | …………….Pg 21 |
| Clock Sync | …………….Pg 21 |
| Daylight Saving | …………….Pg 22 |
| Do Not Disturb | …………….Pg 22 |
| After Hours / Out of Hours | ……….Pg 23 |
| Complete List of Parameters | ……….Pg 23-26 |
| Troubleshooting | ……….Pg 26 |
| Change History | ……….Pg 27 |

Overview of System

Please read this entire manual before attempting to install this system.

This system should only be installed by a professional automatic gate installer or access control specialist dealer.

It is recommended that the system be set up, configured, commissioned and tested on a workshop bench before taken to site for installation.

Site Survey

Before installing this system, you need to be sure that there is good mobile GSM cell coverage in the area it is to be installed. It is recommended that you conduct a site survey, and check reception on the site for a GSM network. If reception is poor in the area, then this system is not recommended.

SIM Card

You will need a SIM card in order to use this system. It should be a regular voice and SMS text SIM card and capable of running on 2G service. Do not use a data only SIM, as this is only for tablets and will not work in the unit.

1) Ensure the SIM has calling credit, and can make and receive calls on a mobile cell phone.

2) Check that the SIM is not locked to a phone and can be used in other devices.

3) Check that the SIM does not have a PIN code request.

4) Disable voicemail service on the SIM.

5) You are now ready to begin programming.

Power

TIP: Most technical calls received are due to installers using CAT5 or alarm cable to power the unit. Neither are rated to carry enough power (2 amp peak). Please use following cable…

| Up to 2 metres (6 feet) – Use minimum 0.5mm2 (18 gauge) Up to 4 metres (12 feet) – Use minimum 0.75mm2 (16 gauge) Up to 8 metres (24 feet) – Use minimum 1mm2 (14 gauge) |  |
| --- | --- |

Using insufficient power cable thickness will cause excessive stress on electronic components, and therefore void the manufacturer’s warranty.

To avoid such problems, it is recommended (and is good practice) to locate the power supply as close to the transmitter as possible. This avoids power cable noise and interference and enhances the lifetime of the product.

Installation

Do not remove the protective film until the system is fully installed and working. Protective coverings are there to protect the intercom from scratches and marks during installation.

Antenna height is best higher than intercom for cleaner audio and also better reception.

Architectural Panels

Hooded Panels

Flush Panels

Tip: Use appropriate fixings to ensure the intercom cannot be removed from the wall.

Inserting the SIM card

Please ensure the SIM card is a 2G compatible Micro SIM card. The SIM may also be 3G and 4G capable as well, as long as both the SIM and the network also support 2G. Do not use a SIM card for a tablet, as these only support data, and do not support voice and SMS. You simply require a mobile phone type SIM card.

Note: This unit is a 2G quad band system, operating on standard 2G network frequencies of 850/900/1800/1900MHz. Ensure the SIM has calling credit, and can make and receive calls on a mobile cell phone.

POWER OFF

45 chamfer OUT.

Copper pads DOWN.

Press until “Click”.

GSM Board

Connections on the GSM Controller

Antenna socket

Use 24vdc PSU

Limit Switch

Exit Button (relay 1)

N/O

COM

N/C

Output 2

Output 1

N/O

COM

N/C

Power IN

24v DC

Use 24v dc PSU

Output 1

Output 2

Output Connections Example

Electric Gates

Relay 1

Relay 2

Other device e.g. exit switch

(not supplied)12v dc

Start

COM

Gate controller

Magnetic lock

-

+

N/C

COM

N/O

COM

This example shows relay 1 connected to a gate motor controller for vehicle gates, and output 2 connected to a magnetic lock for a door or pedestrian gate.

Powering Up

Perform a final check of wiring and ensure the antenna is connected before switching on the power. Once the power is switched on, the power LED should illuminate.

TIPS:

My GSM LED is still searching…

-Check the SIM card is registered and can make a call in a phone.

-Check the SIM card is seated correctly. Power off, clean the contacts on the SIM and the GSM unit, and reinsert the SIM.

-Check power cable distance and thickness.

-Increase antenna height.

-Change network.

-Move antenna away from metal objects or overhanging shrubs.

-Fit a high gain antenna.

Installing the Programmer APP for the first time

1. For android or apple devices you can download the AES programming app called “Cellcom Prime Programmer” (or scan QR code below).

2. Open the app and allow all permissions (Android users).

Press Submit and send your details via email client.

Register your details first time for support registration.

Default Engineer’s and user`s pass codes. These can be changed later.

Enter INTERCOM SIM phone number.

Now you are ready to begin programming!

Enter name or site name for customer.

Programming a Brand-New Install

Press SETTINGS to reveal the screen shown. This screen will store details for the client.

Now you are ready to begin programming!

Programming an EXISTING Install

1.Go to MORE>CLIENT LIST to reveal the screen shown.

2.Press and HOLD to select the desired client.

3.Iphone users press the info symbol. Android users press and hold the client, and then press upload to begin programming.

Programming

Now that you have either entered a new client, or selected an existing client from the client list, you are now ready to begin programming.

Step 1: Check Reception

Go to MORE>INFO & press the reception check button.

On Android the app will automatically send a SMS string (*20#) to the intercom.

On iphone, users will be taken to their SMS screen to confirm before sending the string. The intercom should then reply with a signal level between 1 and 31.

For good performance, signal level on 2G should be at least 13. On 3G products it should be no less than 10.

TIP: If signal is lower than recommended, then take IMMEDIATE action. Change network if possible, or use an optional high gain antenna. Check power cable is within recommended specification. (Poor power cable can lower reception).

Note: SMS string= *20#

SMS reply to your phone.

SMS message sent to intercom.

Step2: Programming Numbers for the intercom to call on button press.

1.Press the SINGLE home icon for a 1 button system, or MULTI for a 10 button system.

2. Simply enter cell phone numbers and/or landline phones which the intercom is to call when the call button is pressed. (10 button model please enter button number).

3. Press SAVE. Note: iphone users will be taken to their SMS screen to confirm the SMS string (press send).

4. The intercom should reply with an SMS to your phone showing the SMS string and an OK status.

SMS Programming Format:

E.g. 9999#111firstnumber#112secondnumber#113thirdnumber#

Programming dial out numbers for multi button versions

5

6

7

7

4

4

4

7

8

8

8

8

3

3

3

3

3

8

9

9

9

2

2

2

2

9

10

10

1

1

Please note the position of the buttons on the above panel options. For example, if you have a 2 button panel, you will be programming dial out numbers for buttons 3 and 8. For a 4 button panel, the corresponding button locations are 2,3,8 and 9.

SMS Programming Method:

Add numbers - 9999#72telephonenumber#72telephonenumber#

Delete specific number –

9999#73telephonenumber#

Delete all –

9999#73*#

To delete, enter number above and press delete

Step3: Programming Caller ID access numbers (100 max).

1.Press the CALLER ID button.

2. Simply enter cell phone numbers of visitors whom should have access with caller ID (up to 8 at a time).

3. Press SAVE. Note: iphone users will be taken to their SMS screen to confirm the SMS string (press send).

4. The intercom should reply with an SMS to your phone showing the SMS string and an OK status.

Programming Additional Features

The intercom should now be able to call users and have some basic Caller ID access. Now you may wish to program additional features for the client, including keypad codes, dialling times (to avoid voicemail on un-answered calls, auto-trigger times etc.

Turn on Auto-clock sync after power failures

Time clock automatic opening and closing times

Turn on SMS notifications when gates triggered.

Programmer and user pass codes

Service Calls (prevent SIM being turned off due to inactivity)

Mic & SPK Volume

Turn on do not disturb to disable call button at night

Adjust, add or delete clients on your client list

Program keypad pin codes

Default relay pulse times

Avoid voicemail picking up un-answered calls

Diagnostics

SMS string for Speaker Volume:

9999#3X#  (X=1-9, default = 5)

SMS string for Microphone Volume:

9999#4X#  (X=1-9, default = 5)

Press SAVE.

TIP: Iphone users will be taken to SMS screen to confirm message. Android devices will automatically send the SMS.

Enter required level (1-9) for optimum speech.

TIP: Set as low as possible for good acoustics.

Default = 5

1.Volumes

Adjust speaker and microphone volumes.

SMS strings:

9999#45XX# (X=dialling time for first number)

9999#46XX# (X=dialling time for second number)

9999#47XX# (X=dialling time for third number)

9999#53XXXX# (X= talking time in seconds, 9999 max)

2. Dialling Times & Talk Time

Change ringing times on each number to avoid voicemail picking up a call on un-answered call so the unit can roll over to the next number.

Note: Default 20 secs (includes 5-8 sec connection time).

Dialling time for second number (default 20 secs)

Dialling time for first number (default 20 secs)

Dialling time for third number (default 20 secs)

Set MAX talking time for all numbers (default 60 secs)

3.Service Calls

This feature is normally only used on intercoms which are seldom used and only for SIM cards which are likely to be de-activated by the network due to inactivity. It can be programmed to make a chargeable outgoing call or SMS to a number of your choice using this screen.

SMS string for choosing SMS or CALL:

9999#58X# (For calls, X=2, for SMS, X=1)

SMS string for entering phone number:

9999#77XXXXXXX# (X=cell phone number) 77*# to delete.

SMS string for frequency of calls:

9999#57XX#

TIP: This will call or SMS at the time at which the feature was activated. So, if you set this feature up at 5pm, it will make the service call or SMS at 5pm at the next interval.

Enter the frequency of calls (1-60 days).

Enter the phone number which is to receive the call

Choose SMS or CALL

TIP: Iphone users will be taken to SMS screen to confirm message. Android devices will auto send the SMS.

4.Info

Check stored phone numbers

Check relay status

Check signal strength

Check stored keypad codes

Check who opened the gate, when it was opened and by what method

|  | SIGNAL STRENTH Will reply with signal range 1-31 Should be higher than 10. RELAY STATUS OPEN – Shows status of the input terminals called STATUS on the intercom, can be used with a limit switch. Relay status shown to check if any relay is latched. STORED CODES NORM=Permanent codes. TEMP=Temporary codes. PLAN=Time restricted codes. STORED NUMBERS O=Dial out number. I=Dial IN Caller ID number. |  |
| --- | --- | --- |

ACTIVITY LOG

Use this to see who used the intercom and when. Which pin codes were used, who used caller ID, who answered the call.

TIP: Time and date is in international military format.

Last 6 digits of caller ID user phone number

Last 6 digits of phone number which answered the gate

Last 2 digits of keypad code used

5. Pass Codes

CAUTION: Take care when changing pass codes. There are 2 levels of 4-digit code (both must be different):

1. Engineers/Programmers code (default 9999)

2. Access/user code (default 1234)

You may wish to change both from their defaults for security.

Enter new user/access code (default 1234)

If changing default codes, then you will now need to update the client list before you can do any further programming.

If the 1234 user access code is changed, then you will also need to change it on the home owners app.

SMS Strings:

9999#01XXXX# (X=new programmers code)

9999#02XXXX# (X=new user access code)

Enter new programmers code (default 9999)

Restore the app to using default codes (does not restore the intercom)

TIP: Iphone users will be taken to SMS screen to confirm message. Android devices will auto send the SMS.

SMS string for relay 1:

9999#50XXXX# (X=time in seconds, 1-9999)

SMS string for relay 2:

9999#51XXXX# (X=time in seconds, 1-9999)

Enter time in SECONDS then press SAVE to send SMS

6.Relay Times

Relay default trigger times are 1 second. Use this feature to change a relay for a longer time perhaps for a magnetic door lock or to make one relay a momentary relay and the other a 1 hour relay for example.

SMS string for turning ON or OFF:

9999#80X# (X=2 to enable. X=1 to disable)

SMS string for entering phone number to receive notification:

9999#78XXXXXXX# (X=cell phone number) 78*# to delete.

SMS string for entering text to display:

9999#79XXXXXXX# (X is any text message you wish to display on the phone. E.g. Gates Opened)

Enter text which you want the user to receive when access is granted, then press SAVE MESSAGE

Enter the phone number to receive the SMS alert and press SAVE

Quick Enable / disable this feature

7.Notifications

This feature is commonly used to allow one home user to receive SMS alerts each time the INTERCOM is used to trigger the gates and grant access.

8.Keypad Programming

For temporary codes (codes which auto-expire in a pre-set time)

For deleting codes

For Time Restricted Codes (codes that work during certain times & days of the week

For Permanent 24/7 Codes

SMS String for adding keypad codes:

Enter activation time. (0-9999 seconds)

Use 1 second for gate trigger.

Use approx. 5 secs for magnetic locks.

Use 0 for latching or toggle code.

Use 3600 secs for 1 hour hold open on gates

Choose Relay 1 or Relay 2

Enter 4 digit code

Permanent Keypad Code

Stores up to 200 codes, all of which can be used to gain access 24/7.

Time (Restricted) Code

Stores up to 20 codes which will only work during pre-set times and days of the week. (Relay 1 only).

SMS String for time restricted code:

Select days of the week the code is to be active.

Enter Start and Finish time

(Military format without any colons.

E.g. 8am = 0800. 11pm = 2300)

Enter 4 digit code

SMS String for temporary code:

Temporary Code

Stores up to 30 codes at any time which will auto expire after a pre-set countdown time (1-168 hours) (Relay 1 only)

Enter 4 digit code

Enter countdown time in hours (1-168 hrs)

SMS String for deleting a known code:

Deleting ALL codes: 9999#84*#

Delete Codes

Delete ALL codes

Delete any known code

You can use these buttons to delete a recently stored temp or time restricted code (codes shown beside the delete button)

9. Auto Relay Trigger Times

Create up to 40 automatic time clock events to trigger or latch/unlatch gates (depending on gate system setup).

For auto closing gates, send latch command at the desired opening time, followed by a separate unlatch event to close at the desired closing time.

For step-by-step operated gates (non auto-closing), then send a momentary trigger command at the time required to change the state of the gates from open to closed or closed to open.

.

3. Enter the time in military 24hr format without colon

E.g. 8.30am = 0830. 11:10pm = 2310.

2. Choose the days

1. Pick the type of event (momentary/latch/unlatch)

Press to select Client

Swipe to delete

Press to load details & program

Press to SAVE after editing

10. Client list on iphone

The client list allows you to save sim phone number, customer name and pass codes for all your installs.

On any previous install, you can load the customer and then re-program their intercom.

Press to load details & begin programming

Press to save changes

Press & HOLD to select Client

10. Client list on android

The client list allows you to save sim phone number, customer name and pass codes for all your installs.

On any previous install, you can load the customer and then re-program their intercom.

11. Clock Sync

The unit has an internal time clock counter, which reads the time from an incoming SMS message, and uses this to calibrate its time clock.

For power failure events, this feature allows the unit to send a SMS to itself after a power failure.

Simply press the button and the app will send a SMS string to the intercom storing the phone number from the SIM card inside memory.

TIP: Use this if your area experiences regular power cuts and your client is using timed features.

Note: Using this feature will cause the unit to be busy for 2-3 minutes after a reboot. Please be patient with programming etc after a re-boot.

TIP: 9999#86*# will delete this number again

Set the number of days between SMS message sending (depending on carrier provider, this may be chargeable to the customer).

11b. Daylight Saving

For regions where there is a 1 hour time shift for daylight saving, it can be useful to have the intercom send itself a SMS every set number of days to re-synchronise the internal clock. The intercom will do this anyway each time a SMS is received.

12. Do not disturb

This feature allows the push button on the intercom to be active during pre-set times, and ignore button presses all other times.

Use this screen to set the ACTIVE times and days for the button.

To activate, enter the following code:

1234#21#ON#  (change ON to OFF to disable again).

Press save to send SMS command.

Select start and finish times for the button to work (24hr format, no spaces or colon. E.g. 8:30am = 0830

Quick enable/disable button

3. Press SAVE to confirm and send SMS

2. Enter button number (enter 1 for single button system)

1.Enter phone number to call after hours.

12b. After Hours / Out of Hours

If you have activated the do not disturb feature, the push button will not call anyone after the pre-set time threshold. However, sometimes it is useful to have the intercom call a different number after hours. For example, in commercial premises it can call the office phones during business hours, and then call a security guard after hours.

Complete list of parameters

The table below show the complete list of features.

| Changing pass codes |  |  |
| --- | --- | --- |
| 9999#01????# | Change programming password | 9999 |
| 9999#02????# | Change access control password (SMS control of relays, or non-stored numbers can call intercom & enter code to activate output 1). | 1234 |
| 9999#03????# | Change monitoring mode password (user can call the intercom, enter this pass code to listen in and speak) | 5555 |
| Dial out numbers |  |  |
| 9999#1XY????# | Store dialling out numbers. (X = button number 1-9 & 0 for button 10) (Y = number dialled 1-4) (???? = phone number) | N/A |
| 9999#1XY*# | Delete a dial out number. (X = button number) (Y = number position 1-4) | N/A |
| Volume controls |  |  |
| 9999#3?# | Speaker volume. Where ? = 1-9. 1 = lowest, 9 = highest. | 5 |
| 9999#4?# | Microphone volume. Where ? = 1-9. 1 = lowest, 9 = highest. | 5 |
| Timings |  |  |
| 9999#50?# | Relay 1 time. ? = seconds, 1-9999 | 1 sec |
| 9999#51?# | Relay 2 time. ? = seconds, 1-9999. | 1 sec |
| 9999#45??# | Calling time for first number, adjust this to avoid voicemail picking up a call (10-99 secs) | 20 secs |
| 9999#46??# | Calling time for second number, adjust this to avoid voicemail picking up a call (10-99 secs) | 20 secs |
| 9999#47??# | Calling time for third number, adjust this to avoid voicemail picking up a call (10-99 secs) | 20 secs |
| 9999#53????# | Talking time. 5-9999 seconds. | 60 secs |
| 9999#55??# | Max monitoring time (for listen in mode when calling the intercom) 00-60 mins. 00 = no limit. | 10 mins |
| Scheduled service calls |  |  |
| 9999# 77number# | Store a service number to receive a scheduled call or SMS from the unit. Useful for SIM cards which are not often used to prevent switch off by the network provider. | N/A |
| 9999#57??# | Set the time schedule for the intercom to make a scheduled call or SMS to the service number. 00-60 day time schedule. 00 = no call or SMS. | 00 |
| 9999#58?# | Choose between making a scheduled call or scheduled SMS. 1 = SMS. 2 = call. | 1 |
| 9999#77*# | Delete the stored service number | N/A |
| Caller ID features |  |  |
| 9999# 72number# | Store caller ID number. Max 14 digits. Only last 6 digits compared. | N/A |
| 9999# 73number# | Delete caller ID number. | N/A |
| 9999#73*# | Delete all caller ID numbers | N/A |
| Service & diagnostic messages (no passcode required for some of these!) |  |  |
| *20# | Check reception level 1-31 (no passcode needed) | N/A |
| *21# | Check stored numbers. O = dial out number. I = dial in number. E = end of message. (no passcode needed) | N/A |
| *22# | Check input status and relay status. (No passcode needed) | N/A |
| *23# | Sends SMS messages of the last 20 events. | N/A |
| 1234#25# | Check stored keypad codes. | N/A |
| Keypad Programming |  |  |
| 9999# 81Xcode#time# | Permanent codes - X=1 or 2 for relay 1 or 2. Code = 4-6 digits. Time = 1-9999 seconds, or 0 for latching code. | N/A |
| 9999# 83#day,day,day #time1,time2# code# | Time restricted codes Day = day of the week e.g. mon,tue,wed,thur,fri. Time1 = start time. Time2 = end time (24 hr format, no colon. E.g. 11:30pm = 2330. 8.30am = 0830. Code = pin code 4-6 digits. | N/A |
| 9999# 82#hours# code# | Temporary codes Hours = time to expire in hours (1-168 hours). Code = Pin code 4-6 digit code. | N/A |
| 9999#84code# | Delete code – Code=known code to be deleted. | N/A |
| 9999#84*# | Delete all codes. | N/A |
| Notifications |  |  |
| 9999#80X# | X=1 to disable. X=2 to enable. | N/A |
| 9999#78XXX# | X=phone number to send notifications to. (*=delete number) | N/A |
| 9999#79text# | X=text to send to the receiving phone e.g. “gate opened” | N/A |
| Automatic Time Clock Trigger Times |  |  |
| 1234#X# day,day,day# time# | X=1,2.3 (trigger, latch, unlatch relay 1) 4,5,6 (relay 2) Day = days of the week (mon,tue,wed,thur,fri,sat,sun) Time = time of day (24 hr format, no colon. E.g. 8:30am = 0830) | N/A |
| 1234*X# | Delete ALL automatic trigger times. | N/A |
| Clock Sync - Auto Time Calibration after Power Fail |  |  |
| 9999#86XXX# | X=telephone number of SIM inside the intercom. | N/A |
| 9999#86*# | Delete the phone number. | N/A |
| Summer Daylight Auto Correct |  |  |
| 9999#87??# | ?? = number of days between SMS calibration SMS should be sent. 0 = no message sending. | N/A |
| Do Not Disturb (push button de-activated during set times) |  |  |
| 1234#21#ON# | ON = activated. OFF = de-activated. | OFF |
| 9999# 21#day,day,day #time1,time2# | Enter all active days during which button should operate. Enter start and end time button should operate (24 hr format, no colon. E.G 8:30am = 0830) | N/A |
| Alternate Number to Call During Do Not Disturb Times. |  |  |
| 9999# 21X????# | X = button number (1-9. Enter 1 for 1 button system. Enter 0 for button 10) ???? = Alternative phone number to call out of hours. |  |
| Restore Defaults |  |  |
| 9999#999# | Send with passcode string to clear all programming. | N/A |

Troubleshooting

Please see faults in order of most common…

1: Not detecting network (blue light flashing 5 times in search mode, no green CPU light).

A: SIM card not detected. Power off, remove, clean sim contacts and re-insert and power on again.

A: SIM card inserted wrong way round. Check manual for correct orientation.

A: SIM is a data sim, or has not been activated.

A: SIM has no signal in the area.

A: Antenna not connected.

A: Too many sharp bends on antenna cable.

A: Antenna mounted too low or inside metal enclosure.

A: Power cable from 24v psu is too long or too thin. Refer to manual for guidelines.

2: Not responding to SMS messages and not making outgoing calls.

A: No credit on SIM card.

A: Power cable not within spec. Refer to manual for guidelines.

3: Not triggering gates or lock when activated from phone.

A: Check relay with multi-meter.

A: Check relay 2 with multi-meter. If relay 2 works but relay 1 does not, then relay 1 may be defective.

A: Check power cable is within specifications according to this manual.

A: Check if it works by SMS. Try latching a relay then use the status button to check if the relay is latched. If that works, problem could be the phone being used, or low signal strength at the intercom.

3: Poor sound or buzzing

A: Ensure there is no spare antenna cable inside the call point with the intercom. Straighten excess cable.

A: Install the antenna further away from the intercom.

A: Check the power cable is within guidelines of this manual.

4: Home owner app not working correctly

A: Check the settings on the app has the intercom SIM number and pass codes entered correctly.

Change History

Key:

P = Panel version    H = Hardware PCB version     S = Software version

| Version |  |  | Reason for change |
| --- | --- | --- | --- |
| P | H | S |  |
| 1 | 1 | 1 | First version. |
| 1 | 2 | 1 | Power chip upgraded to work on 24v dc (24v adaptor in kits). |
| 1 | 2 | 2 | Software feature added for call log to show last 25 caller ID calls. |
| 1 | 3 | 2 | Main capacitor, regulator & diode upgraded for 24v ac. |
| 1 | 4 | 2 | Micro Sim Holder and complete board respin |
| 1 | 4 | 3 | Caller ID fix for international + symbol. (October 2016) |
| 1 | 5 | 4 | PRIME model, new firmware plus larger flash storage. (November 2016) |
| 1 | 6 | 4 | SMA connection directly on the board, micro sim and 24v PSU (July 2017) |